- Command to initiate sending the data to the server in a certain interval of time.

SetUploadFreq <data0>,<data1>

where 	<data0> = meter number (1 or 2)

<data1> = integer value ranging from 1 to 3600

LxSetUploadFreq <data0>,

where <data0> = success or failure (0 or 1)

- Spontaneous data upload format

<data0>,<data1>,<data2>,<data3>

Where

<data0> = timestamp

<data1> = meter number (1 or 2)

<data2> = totalizer value

<data3> = IMEI number

- Commands to replace the need for ez-command tool

- Following settings need to be addressed via pandabox

a. No-Flow timer

The following command is used to query No flow timer

7E 7E 01 14 01 02 20 19 6C CD

7E 7E 02 14 01 02 20 19 B0 56

and the response for lcp node 1 is

7E 7E 14 01 81 04 00 31 00 64 DA 7F 0D 0A  where  00 64 indicate 100 seconds no-flow timer set in the meter

But we now need to set the no-flow timer where the timer value can take 0 to 3600 decimal values.

b. Ticket required condition

the following command is used to query the ticket required condition

7E 7E 01 14 01 02 20 25 50 CD

7E 7E 02 14 01 02 20 25 8C 56

And the response for LCP node 1 is

7E 7E 14 01 81 03 00 31 00 07 E0 0D 0A  ticket required YES 00

7E 7E 14 01 81 03 00 31 01 06 E0 0D 0A  ticket required NO  01

7E 7E 14 01 81 03 00 31 02 05 E0 0D 0A  ticket required SKIP  02

again, we need to set the ticket required condition to '00', '01' or '02' with '01' mostly used.

c. Preset Type used

Currently I could not generate the command to query the Preset Type used.

Generally, these are the basic configurations we need to change in the LCR meters using the ez command tool.

If we could change them using the pandabox commands, that would be very helpful.

2. Implementing commands for ezcommand status

a. RdEzCmdStatus <data0>,

where <data0> - port number

the response should look like,

LxRdEzCmdStatus <data0>, <data1>, <data2>,<data3>

where

<data0> - port number;  1 or 2

<data1> - LCP node address of the meter connected to the port

<data2> - no-flow timer of the meter

<data3> - ticket required condition

<data4> - preset type

# LCP node address can range from 1 to 250

# no-flow timer can range from 0-3600

# ticket required condition can be

0 - yes

1 - no

2 - skip

# preset type can be

clear

multiple

retain

3. Implementing commands to Set ezcommand status

- SetNoFlow <data0>,<data1>

Where,

<data0> = Port number 1 or 2

<data1> = 0 to 3600 integer value

Return should be

LxSetNoFlow <data0>

Where,

<data0> = 0 for success and 1 for failure

- SetTicketReqd <data0>,<data1>

Where,

<data0> = Port number 1 or 2

<data1> = 1 (to change ticket required to ‘No’)

Return should be

LxSetTicketReqd <data0>,

Where,

<data0> = 0 for success and 1 for failure

- SetPresetType <data0>,<data1>

Where,

<data0> = Port number 1 or 2

<data1> = clear or multiple or retain

Return should be

LxSetPresetType <data0>,

Where,

<data0> = 0 for success and 1 for failure