Panada-Box Communication Protocol

Rev 1.86

PCL TECHNOLOGY CO., LIMITED

December, 2025

catalogue

f

LCRBOX FLASH Storage Structure	2

1. Read storage information, history data	3

1.1 Query history record time	3

1.2 Query BOX storage status	3

1.3 Read the stored data	3

1.4 Feedback the received data results to the box	4

1.5 Read the historical data by timestamp	5

2. Delete storage card data	6

2.1 Delete all the storage data from one meter	6

3. Bluetooth/WIFI/TCP Server Parameters	6

3.1 Read the current Bluetooth name	6

3.2 Set the Bluetooth name	6

3.3 Set the Bluetooth Password	7

3.4 Set WiFi name	7

3.5 Set WiFi Password	7

3.6 TCP/IP Server IP address Set	7

3.7 TCP/IP Server Port Set	7

3.8 APN Set	8

4. Configure BOX parameters, inquiry status	8

4.1  Read the box current setting status, stored meter number, current time, GPS coordinates, network status	8

4.2 Set Box connection mode	8

4.3 Read the box device version information, hardware version, date, software version, and date	8

4.4 BOX Reset and start	9

4.5 Set BOX time	9

4.6 RS485/RS232 mode switch	9

SetRs485 0/1   1 means switch to RS485, 0 means switch to RS232	9

4.7 Set Start	9

SetApp1 set start from APP1	9

SetApp2 set start from APP2	9

5. Meter communication control	9

5.1 Start communicate	9

5.2 Pause communicate	9

5.3 Stop communicate	9

5.4 Print communicate	10

6. New commands	10

6.1 Set the meter address of the corresponding port	10

6.2 Read the meter address of the corresponding port	10

6.3 Modify meter address	10

6.4 Set the meter pre-set gallon value	11

6.5 Return Box current time	11

6.6 Query the meter connection status	11

6.7 Query the meter current working status	11

6.8 Query the meter address number	11

7. TCS commands	11

Version: 1.86

History modification:

1.3: 2023/6/2 Add the totalizator values.

1.4:2023/7/2 Add new commands (set/read the meter address of the corresponding port).

1.5: 2023/7/23 Add the meter pre-set gallon value, Add the odometer of the meter.

1.6 :2023/8/12 Add the blue content.

1.7:2023/8/26 Modify the time as UNIX time, add the identification of real time meter data and history meter data reporting.

1.8: 2023/10/24 Modify the query historical record time, add GPS information in the real time data and historical data, query historical data method, Query meter working status, meter connection status, query meter address number etc.

1.82: Modify the history data report starts with LxGetDataTs, Modify all the command end with “,”

1.83: 2023/11/11 Add bluetooth password set/modify, WiFi user/password set/modify

1.84: 20223/12/4 Add print command, Set server IP/Port Commands

1.85: 2024/1/27 Add the APN configuration command

1.86: 2025/11/04 Modify 4.2 set box connection mode, 4.6 RS485/RS232 switch, 4.7 set start, 7. TCS commands

# LCRBOX FLASH Storage Structure

The storage is divided into two zones, which can store two meters data at the same time. When not connected to the APP, it will store the data circularly and the earliest data will be deleted when the storage is full.

When replacing the connected meter with BOX, if the communication is successful and the meter number changes, the stored data of the previously connected meter will be deleted. Therefore, if the meter needs to be replaced, please import the meter data stored in BOX into the APP before replacement. Otherwise, after successfully connecting to the new meter communication, the original meter data will be overwritten by the new meter data

Transmission packet format: (The command starts with uppercase ASCII characters and ends with<CR><LF>. CR-Carriage Return, LF-Line Feed means carriage returns and line feeds. The data is separated by commas, and both uplink and downlink commands are in this format, the last parameter also ends with a comma, which is convenient for unified processing)

APP->BOX, Xxxxx:Command word, according to the command word determines the format of the following data

BOX->APP, LxXxxxx:Returns command word, BOX returns the command word, and adds Lx before the APP request command word. For example, if the request command number is DataTime, then return LxDataTime, and the format of the subsequent data will be determined based on the command word

The communication protocol definition between APP and LCRBOX:

- Read storage information, history data

## 1.1 Query history record time

Command number HisDataTime <data0>,

Parameter byte<data0>  meter number

Return field LxHisDataTime <data0>, <data1>

<data0> meter number

<data1>  The time of the earliest historical data, UNIX time format

<data2>  The time of the latest historical data, UNIX time format

For example: APP->BOX     HisDataTime 1,<CR><LF>

Box->App   LxHisDataTime 1,1695875000，1695875398，<CR><LF>

(The earliest time is 1695875000, the latest time is 1695875398)

## 1.2 Query BOX storage status

The stored data counts and remaining storage space from each meter

Command number BoxStorage <data0>

<data0> meter number；

Returned command number LxBoxStorage <data0>, <data1>, <data2>

<data0> meter number

<data1> Stored data counts

<data2> Remaining storage space

## 1.3 Read the stored data

Read the stored data at the specified time, return the earliest stored data if no time is specified

Command number GetData <data0>,<data1>

<data0> meter number, Box finds the port number to communicate with the meter according to the meter address

<data0> 0: real time meter data report, 1: history meter data report (Reporting start from the earliest stored data in the storage area)

Return command number LxGetData <data0>, <data1> ,<data2> ,<data3>, <data4> ...

Note: if real time meter data report starts with LxGetData, if history meter data report starts with LxGetDataTs

<data0> meter number

<data1>Data length, this package contains several sets of data n. the length is 0 if there is no data in the storage area, and there are no sequential content fields

<data2> Pack serial number, increase in order, and start from 0 after adding up to 255

<data3> data time 1

<data3> data time 1  UNIX time format

<data4> data content 1a (data flow)

<data5> data content 1b (totalizator values)

<data6> data content 1c (data flow 17)

<data7> data content 1d (data flow 18)

<data8> data content 1e (data flow 100)

<data9> data content 1f (data flow 101)

<data10> data content 1g       (GPS longitude)

<data11> data content 1h       (GPS latitude)

...

<data n> data time n

<data n+1> data content na (data flow)

<data n+2> data content nb (totalizator values)

<data n+3> data content nc (data flow 17)

<data n+4> data content nd (data flow 18)

<data n+5> data content ne (data flow 100)

<data n+6> data content nf (data flow 101)

<data n+7> data content 1g       (GPS longitude)

<data n+8> data content 1h       (GPS latitude)

## 1.4 Feedback the received data results to the box

After the BOX sends a stored data package, the APP will return a successful reception flag after receiving it. The BOX will continue to upload the next data package based on the successful reception flag. If the successful reception flag is not received, it will stop to upload. The storage card will delete it after returning the APP successfully receives this data package.

Command number  GetDataEcho <data0>, <data1>,<data2>,

<data0> Packet serial number, BOX determines whether the uploaded data packet corresponds to the previous one based on its serial number. If it is inconsistent, it is also considered as an upload failure. The APP needs to reissue the GetData command to retrieve the data

<data1> 0： Execution successful 1：Execution Fail

<data2> Request the number of data records

Returned command number  LxGetDataTs <data0>, <data1> ,<data2> ,<data3>, <data4> ...

<data0> meter number

<data1> Data length, this package contains several sets of data n. the length is 0 if there is no data in the storage area, and there are no sequential content fields

<data2> Pack serial number, increase in order, and start from 0 after adding up to 255

<data3> data time 1

<data4> data content 1a (data flow)

<data5> data content 1b (totalizer values)

<data6> data content 1c (data flow 17)

<data7> data content 1d (data flow 18)

<data8> data content 1e (data flow 100)

<data9> data content 1f (data flow 101)

<data10> data content 1g       (GPS longitude)

<data11> data content 1h       (GPS latitude)

...

<data n> data time n

<data n+1> data content na (data flow)

<data n+2> data content nb (totalizer values)

<data n+3> data content nc (data flow 17)

<data n+4> data content nd (data flow 18)

<data n+5> data content ne (data flow 100)

<data n+6> data content nf (data flow 101)

<data n+7> data content 1g       (GPS longitude)

<data n+8> data content 1h       (GPS latitude)

## 1.5 Read the historical data by timestamp

Command number GetDataTs <data0>, <data1>, <data2>,

<data0> – meter address number

<data1> – the historical data start time (UNIX time 1)

<data2> – the historical data end time （UNIX time 2）

(Because the Bluetooth transmission data is too long, it will be subcontracted. In order to avoid the amount of data is too large from one data package, the multiple packets will be transmitted separately)

Returned command number:

LxGetDataTs <data0>, <data1>,<data2>,<data3>...

LxGetDataTs <data0>, <data1>,<data2>,<data3>...

LxGetDataTs <data0>, <data1>，<data2>,<data3>...

...

LxGetDataTs <datan>,

<data0> meter number

<data1> Pack serial number, from 1 to 100

<data2> data time, UNIX format

<data3> data content 1a (data flow)

<data4> data content 1b (totalizer values)

<data5> data content 1c (data flow 17)

<data6> data content 1d (data flow 18)

<data7> data content 1e (data flow 100)

<data8> data content 1f (data flow 101)

<data9> data content 1g       (GPS longitude)

<data10> data content 1h       (GPS latitude)

- Delete storage card data

## 2.1 Delete all the storage data from one meter

Command number DeleteAll <data0>

<data0> meter number, If the device number does not correspond to the device number which stored the data in the BOX, then no action will be performed

Returned command number  LxDeleteAll <data0>

<data0> 0:Execution success 1:Execution Fail

- Bluetooth/WIFI/TCP Server Parameters

## 3.1 Read the current Bluetooth name

Command number RdBtName

Returned command number LxRdBtName <data0>

<data0> The Bluetooth name string, maximum 8 bytes

## 3.2 Set the Bluetooth name

Command number  SetBtName <data0>

<data0> The Bluetooth name string, maximum 16 bytes

Return command number LxSetBtName <data0>

<data0> 0:Execution success 1:Execution Fail

After writing the Bluetooth name to the BOX device, the BOX needs to restart the Bluetooth module then the Bluetooth name will be updated. The mobile app needs to wait for a certain time before re-searching for the Bluetooth device and reconnecting

## 3.3 Set the Bluetooth Password

Command number SetBtPwd<data0>

<data0> bluetooth password string, max 4 bytes

Return command number LxSetBtPwd<data0>

<data0> 0:Execution success 1:Execution Fail

## 3.4 Set WiFi name

Command number  SetWifiName <data0>

<data0> WIFi name string, max 16 bytes

Returned command number  LxSetWifiName <data0>

<data0> 0:Execution success 1:Execution Fail

## 3.5 Set WiFi Password

Command number: SetWifiPwd <data0>

<data0> WFIFI password string, max 16 bytes

Returned command number:  LxSetWifiPwd <data0>

<data0> 0:Execution success 1:Execution Fail

After the box device is written to the wifi Bluetooth name, the box needs to be restarted to take effect, and the phone app has to wait for some time to search the device again before reconnecting

## 3.6 TCP/IP Server IP address Set

Command number: SetServerIp <data0>

data0, server IP address

Returned command number LxSetServerIp<data0>

<data0> 0:Execution success 1:Execution Fail

## 3.7 TCP/IP Server Port Set

Command number: SetServerPort <data0>

<data0>, server port

Returned command number LxSetServerPort<data0>

<data0> 0:Execution success 1:Execution Fail

## 3.8 APN Set

Command number: SetApn <data0>

<data0>, APN

Returned command number LxSetApn<data0>

For example: SetApn cment  (cmnet is the APN)

<data0> 0:Execution success 1:Execution Fail

- Configure BOX parameters, inquiry status

## 4.1  Read the box current setting status, stored meter number, current time, GPS coordinates, network status

Command number BoxStatus

Returned command number LxBoxStatus <data0>, <data1>, <data2>, <data3> ,<data4> <data5> ,<data6>,<data7> ,<data8>

<data0> 1: bridge connecting (Real-time transmit); 2: LCR analyze storage

<data1> Box working status, reserved bytes, specific definitions will be added based on the subsequent functions

<data2> meter number 1

<data3> meter number 2

<data4> current time, UNIX format

<data5> GPS coordinate longitude

<data6> GPS coordinate latitude

<data7> Current network status, 4g network status, 1 means normal connection, 2 means connection failure

<data8> Status of wifi and bluetooth, 0: no connection to app, 1:Bluetooth connection normal, 2: WiFi connection normal

## 4.2 Set Box connection mode

Command number SetMode <data0>

<data0> set parameter 1: bridge connecting (Real-time transmit)  2: cmd  3: print (Print mode only accepts information for printing by the app )

Returned command number LxSetMode <data0>

<data0> 0: Execution succeeded 1: Execution failed

## 4.3 Read the box device version information, hardware version, date, software version, and date

Command number BoxInfo

Returned format：

Returned command number LxBoxInfo <data0>, <data1>, <data2>, <data3>

<data0> hardware version, 5 bytes ASCII code character(xx.xx)

<data1> date, ddhhmm

<data2> software version, 5 bytes ASCII code character (xx.xx)

<data3> date ddhhmm

## 4.4 BOX Reset and start

Command number BoxReset,BOX soft reset, restart

Returned command number LxBoxReset <data0>  Because the BOX needs to take some time to restart, it will return the receiving status first and then restart.

During the restarting, some modules will have a delay starting and will not receive APP commands for a few seconds, Bluetooth needs to reconnect.

<data0> 0: Execution succeeded 1: Execution failed

## 4.5 Set BOX time

Command number SetBoxTime <data0>

<data0> Set time UNIX format

Returned command number LxSetBoxTime <data0>

<data0> 0: Execution succeeded 1: Execution failed

## 4.6 RS485/RS232 mode switch

## SetRs485 0/1   1 means switch to RS485, 0 means switch to RS232

## 4.7 Set Start

## SetApp1 set start from APP1

## SetApp2 set start from APP2

- Meter communication control

  - Start communicate

Command number Start <data0>

<data0> meter address number

- Returned command number LxStart <data0>

<data0> 0: Execution succeeded 1: Execution failed

  - Pause communicate

Command number Pause <data0>

<data0> meter address number

- Returned command number LxPause <data0>

<data0>0: Execution succeeded 1: Execution failed

  - Stop communicate

Command number Stop <data0>

<data0> meter address number

- Returned command number LxStop <data0>

<data0> 0: Execution succeeded 1: Execution failed

  - Print communicate

Command number Print <data0>

<data0> meter address number

- Returned command number LxPrint <data0>

<data0> 0: Execution succeeded 1: Execution failed

- New commands

## 6.1 Set the meter address of the corresponding port

(The port which straight to the LED as port 1, the other port as port 2)

Command number  SetPortLcrNode <data0>, <data1>，

<data0> The meter number of the corresponding port 1, set it as 0 if there is no meter connection (1~255)

<data1> The meter number of the corresponding port 2, set it as 0 if there is no meter connection (1~255), end with ,

Returned command number:  LxSetPortLcrNode <data0>

<data0> Execution succeeded 1: Execution failed

## 6.2 Read the meter address of the corresponding port

(The port which straight to the LED as port 1, the other port as port 2）

Command number:  RdPortLcrNode

Returned command number:  LxRdPortLcrNode <data0>, <data1>

<data0> The meter number of the corresponding port 1, set it as 0 if there is no meter connection (1~255)

<data1> The meter number of the corresponding port 2, set it as 0 if there is no meter connection (1~255)

## 6.3 Modify meter address

Command number:  ModifyLcrNode <data0>,<data1>,<data2>，

<data0> Port number (1~2)

<data1> The original meter number

It will return error if it doesn’t correspond to the actual meter number

<data2> Set the new meter number (1~255)

Returned command number:  LxModifytLcrNode <data0>

<data0> 0: Execution succeeded 1: Execution failed

## 6.4 Set the meter pre-set gallon value

Command number: PresetGross <data0>,<data1>,

<data0> port number（1~2）

<data1> Preset gallon value (Floating number, with one decimal)

Returned command number:  LxPresetGross <data0>

<data0> 0: Execution succeeded 1: Execution failed

Example:  PresetGross 1,1234.5,

## 6.5 Return Box current time

Command number: BoxTime

Returned command number: LxBoxTime <data0>,<data1>,

<data0> Box current time, UNIX format

<data1> GPS coordinate,string format

## 6.6 Query the meter connection status

Command number:  RdRegister

Returned command number: LxRdRegister <data0>,<data1>,

<data0>:    ‘0’ : the meter is not register,  ‘1’ : port 1 register the meter.

<data1>:    ‘0’ : the meter is not register,   ‘1’ : port 2 register the meter

## 6.7 Query the meter current working status

Command number: SwitchState <data0>,

<data0>: 1 or 2 ( means the port 1 or port 2).

Returned command number: LxSwitchState <data0>,<data1>,

<data0> : meter address number

<data1>:  meter working status (Run/ Stop/ Print/ Shift Print)

## 6.8 Query the meter address number

Command number:  GetLcrNode <data0>,<data1>,<data2>

<data0>: 1 or 2 (port number)

<data1>: 1~250 (Starting lcp node address number)

<data2>: 1~250 (End of the lcp node address number)

Returned command number: LxFindLcrNode <data0>,<data1>,

<data0>: 1 or 2 (port number)

<data1>: 1~250 (LCP Node address number)

- TCS commands

DirectDelivery 2,5 configure the address to meter No. 2  directly, Directly configure the address as meter product ID 1009+5 for meter No.2

PresetGross 2,5,125.3 The reservation configuration address is for meter product ID 1009+5, with a reservation value of 125.3

{0x1c, "LxGetSysTotalNet"},

{0x1e, "LxGetSysTotalGross"},

{0x2b, "LxGetDisplayGross"},

{0x10, "LxGetProdTotalGross"},

{0x11, "LxGetProdTotalNet"},

{0x42, "LxGetFlowrate"},

{0x2c, "LxGetDisplayNet"},

{0x2d, "LxGetDisplayVol"},

{0x1f, "LxGetDeliveryState"},

{0x35, "LxGetStatusShift"},

{0x39, "LxPause"},

{0x3a, "LxResume"},

{0x3b, "LxAbort"},

{0x3d, "LxStop"},

{0x3e, "LxPrint"},

{0x1a, "LxGetNoFlow"},

{0x3c, "LxStart"},//Start 2